// hello.rxtx.44.c

//

// step response transmit-receive hello-world

//    115200 baud FTDI interface

//

// Neil Gershenfeld

// 11/6/11

//

// (c) Massachusetts Institute of Technology 2011

// This work may be reproduced, modified, distributed,

// performed, and displayed for any purpose. Copyright is

// retained and must be preserved. The work is provided

// as is; no warranty is provided, and users accept all

// liability.

//

#include <avr/io.h>

#include <util/delay.h>

#define output(directions,pin) (directions |= pin) // set port direction for output

#define set(port,pin) (port |= pin) // set port pin

#define clear(port,pin) (port &= (~pin)) // clear port pin

#define pin_test(pins,pin) (pins & pin) // test for port pin

#define bit_test(byte,bit) (byte & (1 << bit)) // test for bit set

#define bit_delay_time 8.5 // bit delay for 115200 with overhead

#define bit_delay() _delay_us(bit_delay_time) // RS232 bit delay

#define half_bit_delay() _delay_us(bit_delay_time/2) // RS232 half bit delay

#define char_delay() _delay_ms(10) // char delay

#define PWM_delay() _delay_us(25) // PWM delay

#define nloop 100 // loops to accumulate

#define serial_port PORTA

#define serial_direction DDRA

#define serial_pin_out (1 << PA4)

#define transmit_port PORTA

#define transmit_direction DDRA

#define transmit_pin (1 << PA2)

#define led_port PORTA

#define led_direction DDRA

#define red (1 << PA7)

#define green (1 << PA6)

#define led_port PORTB

#define led_direction DDRB

#define blue (1 << PB2)

void put_char(volatile unsigned char *port, unsigned char pin, char txchar) {

//

// send character in txchar on port pin

//    assumes line driver (inverts bits)

//

// start bit

//

clear(*port,pin);

bit_delay();

//

// unrolled loop to write data bits

//

if bit_test(txchar,0)

set(*port,pin);

else

clear(*port,pin);

bit_delay();

if bit_test(txchar,1)

set(*port,pin);

else

clear(*port,pin);

bit_delay();

if bit_test(txchar,2)

set(*port,pin);

else

clear(*port,pin);

bit_delay();

if bit_test(txchar,3)

set(*port,pin);

else

clear(*port,pin);

bit_delay();

if bit_test(txchar,4)

set(*port,pin);

else

clear(*port,pin);

bit_delay();

if bit_test(txchar,5)

set(*port,pin);

else

clear(*port,pin);

bit_delay();

if bit_test(txchar,6)

set(*port,pin);

else

clear(*port,pin);

bit_delay();

if bit_test(txchar,7)

set(*port,pin);

else

clear(*port,pin);

bit_delay();

//

// stop bit

//

set(*port,pin);

bit_delay();

//

// char delay

//

bit_delay();

}

int main(void) {

//

// main

//

static unsigned char count;

static uint16_t up,down;

//unsigned char count, pwm;

//

// set clock divider to /1

//

CLKPR = (1 << CLKPCE);

CLKPR = (0 << CLKPS3) | (0 << CLKPS2) | (0 << CLKPS1) | (0 << CLKPS0);

//

// initialize output pins

//

set(serial_port, serial_pin_out);

output(serial_direction, serial_pin_out);

clear(transmit_port, transmit_pin);

output(transmit_direction, transmit_pin);

//

// initialize LED pins

//

set(led_port, red);

output(led_direction, red);

set(led_port, green);

output(led_direction, green);

set(led_port, blue);

output(led_direction, blue);

// init A/D

//

ADMUX = (0 << REFS2) | (0 << REFS1) | (0 << REFS0) // Vcc ref

| (0 << ADLAR) // right adjust

| (0 << MUX3) | (0 << MUX2) | (1 << MUX1) | (1 << MUX0); // PB3

ADCSRA = (1 << ADEN) // enable

| (1 << ADPS2) | (1 << ADPS1) | (1 << ADPS0); // prescaler /128

//

// main loop

//

while (1) {

//

// accumulate

//

up = 0;

down = 0;

for (count = 0; count < nloop; ++count) {

//

// settle, charge

//

settle_delay();

set(transmit_port, transmit_pin);

//

// initiate conversion

//

ADCSRA |= (1 << ADSC);

//

// wait for completion

//

while (ADCSRA & (1 << ADSC))

;

//

// save result

//

up += ADC;

//

// settle, discharge

//

settle_delay();

clear(transmit_port, transmit_pin);

//

// initiate conversion

//

ADCSRA |= (1 << ADSC);

//

// wait for completion

//

while (ADCSRA & (1 << ADSC))

;

//

// save result

//

down += ADC;

}

//

// send framing

//

put_char(&serial_port, serial_pin_out, 1);

char_delay();

put_char(&serial_port, serial_pin_out, 2);

char_delay();

put_char(&serial_port, serial_pin_out, 3);

char_delay();

put_char(&serial_port, serial_pin_out, 4);

//

// send result

//

put_char(&serial_port, serial_pin_out, (up & 255));

char_delay();

put_char(&serial_port, serial_pin_out, ((up >> 8) & 255));

char_delay();

put_char(&serial_port, serial_pin_out, (down & 255));

char_delay();

put_char(&serial_port, serial_pin_out, ((down >> 8) & 255));

char_delay();

//

// off -> red

//

for (count = 0; count < 255; ++count) {

clear(led_port,red);

for (pwm = count; pwm < 255; ++pwm)

PWM_delay();

set(led_port,red);

for (pwm = 0; pwm < count; ++pwm)

PWM_delay();

}

//

// red -> green

//

for (count = 0; count < 255; ++count) {

set(led_port,red);

clear(led_port,green);

for (pwm = count; pwm < 255; ++pwm)

PWM_delay();

clear(led_port,red);

set(led_port,green);

for (pwm = 0; pwm < count; ++pwm)

PWM_delay();

}

//

// green -> blue

//

for (count = 0; count < 255; ++count) {

set(led_port,green);

clear(led_port,blue);

for (pwm = count; pwm < 255; ++pwm)

PWM_delay();

clear(led_port,green);

set(led_port,blue);

for (pwm = 0; pwm < count; ++pwm)

PWM_delay();

}

//

// blue -> on

//

for (count = 0; count < 255; ++count) {

set(led_port,blue);

clear(led_port,green);

clear(led_port,red);

for (pwm = count; pwm < 255; ++pwm)

PWM_delay();

set(led_port,blue);

set(led_port,green);

set(led_port,red);

for (pwm = 0; pwm < count; ++pwm)

PWM_delay();

}

//

// on -> off

//

for (count = 0; count < 255; ++count) {

set(led_port,blue);

set(led_port,green);

set(led_port,red);

for (pwm = count; pwm < 255; ++pwm)

PWM_delay();

clear(led_port,blue);

clear(led_port,green);

clear(led_port,red);

for (pwm = 0; pwm < count; ++pwm)

PWM_delay();

}

}

}